# UVIS Image Cube (REVISED July 11, 2007)

Each observation is organized as a series of 2-d spatial image planes, spatial pixel by readout. The cube is formed by creating an image plane for each spectral bin. Thus the first 1024, 512, 341, or 256 (or other spectral binning that has yet to be used) image layers will be UVIS data. Geometry data is calculated for the mid-point time of each integration. The concept of the UVIS image cube is similar to that of standard hyperspectral image cubes (see AVIRIS data, for example) but with the addition of geometric information included as additional planes.

The image cubes are saved as binary files, with network byte order (Intel based). The cube dimensions are arranged as follows.

Image_cube = fltarr(spatial dimension, readouts, spectral)

The UVIS data in the image planes have been flatfield calibrated using Andrew Steffl’s routines, the time-variant Red Patch as derived by Holtzclaw and McClintock, and had an empirically derived Radioisotopic Thermal Generator (RTG) noise component subtracted at a constant rate of 4e-6 counts/second/pixel.

In addition to the image data planes, the geometric data is included in the last 49 planes. The order of the geometric data planes are as follows, with the Plane # being the number of the plane after the image data (ie. 1024 + 1,2,3, etc.).

UVIS data cubes are also available as IDL save files. These save files restore a structure that includes all the data as the image cube, ‘datastruct’, but with slightly different naming conventions that take advantage of IDL structures. See the document, UVISImageCube_descriptions.doc, included in the Cube Generator distribution for further details about the structure formats.

Plane # 	Quantity					Geometer Source

1		Pixel Center RA				data.ra_full(0,*)

2		Pixel Lower Left RA				data.ra_full(1,*)

3		Pixel Upper Left RA				data.ra_full(2.*)

4		Pixel Upper Right RA				data.ra_full(3,*)

5		Pixel Lower Right RA			data.ra_full(4,*)

6		Pixel Center Declination			data.dec_full(0,*)

7		Pixel Lower Left Declination			data.dec_full(1,*)

8		Pixel Upper Left Declination			data.dec_full(2.*)

9		Pixel Upper Right Declination		data.dec_full(3,*)

10		Pixel Lower Right Declination		data.dec_full(4,*)

11		Pixel Center Latitude				data.lat_full(0,*)

12		Pixel Lower Left Latitude			data.lat_full(1,*)

13		Pixel Upper Left Latitude			data.lat_full(2.*)

14		Pixel Upper Right Latitude			data.lat_full(3,*)

15		Pixel Lower Right Latitude			data.lat_full(4,*)

16		Pixel Center Longitude			data.lon_full(0,*)

17		Pixel Lower Left Longitude			data.lon_full(1,*)

18		Pixel Upper Left Longitude			data.lon_full(2.*)

19		Pixel Upper Right Longitude			data.lon_full(3,*)

20		Pixel Lower Right Longitude			data.lon_full(4,*)

21		Pixel Center Incidence Angle			data.incidence

22		Pixel Center Emission Angle			data.emission

23		Pixel Center Phase Angle			data.phase

24 		Pixel Center Rayheight			data.rayheight

25		Pixel Center Occultation Latitude		data.latocc

26 		Pixel Center Occultation Longitude		data.lonocc

27 		Pixel Center Line of Sight Distance		data.losdist

28		Sub-Spacecraft Longitude			data.subsclon

29 		Sub-Spacecraft Latitude			data.subsclat

30 		Sub-Solar Longitude				data.subsolarlon

31		Sub-Solar Latitude				data.subsolarlat

32		Spacecraft Altitude				data.alt

33 		Target Right Ascension			data.ratarg

34 		Target Declination				data.dectarg

35		Target Phase Angle				data.phasetarg

36		Target Incidence Angle			data.incidencetarg

37		Target Emission Angle			data.emissiontarg

38		Spacecraft Location, X			data.scloc(0)

39		Spacecraft Location, Y			data.scloc(1)

40		Spacecraft Location, Z			data.scloc(2)

41		Spacecraft Velocity, X			data.scvel(0)

42		Spacecraft Velocity, Y			data.scvel(1)

43		Spacecraft Velocity, Z			data.scvel(2)

44		ET						data.et

45		Center Ring Plane Radii			data.rrad_full(0)

46		Lower Left Ring Plane Radii			data.rrad_full(1)

47		Upper Left Plane Radii			data.rrad_full(2)

48		Upper Right Plane Radii			data.rrad_full(3)

49		Lower Right Plane Radii			data.rrad_full(4)

The Following backplanes are only written when the image cube is output as an IDL save file, in which case it is a string vector equal in length to the number of readouts.

50		UTC						time

51 		kernels						Memory loaded SPICE kernels