This document describes in greater detail the geometric data included in the backplanes, or associated structure, of the UVIS image cube. See the figures for further information about what geometric information each structure name represents.

Plane # 	Structure Name					Geometer Source

1		Pixel_Center_RA					data.ra_full(0,*)

2		Pixel_Lower_Left_RA				data.ra_full(1,*)

3		Pixel_Upper_Left_RA				data.ra_full(2.*)

4		Pixel_Upper_Right_RA				data.ra_full(3,*)

5		Pixel_Lower_Right_RA				data.ra_full(4,*)

These are the Right Ascension coordinates of the pointing vector for the center and corners of each pixel.

6		Pixel_Center_Declination				data.dec_full(0,*)

7		Pixel_Lower_Left_Declination			data.dec_full(1,*)

8		Pixel_Upper_Left_Declination			data.dec_full(2.*)

9		Pixel_Upper_Right_Declination			data.dec_full(3,*)

10		Pixel_Lower_Right_Declination			data.dec_full(4,*)

Similarly, this is the Declination coordinate of the center and corners of each pixel

11		Pixel_Center_Latitude				data.lat_full(0,*)

12		Pixel_Lower_Left_Latitude				data.lat_full(1,*)

13		Pixel_Upper_Left_Latitude				data.lat_full(2.*)

14		Pixel_Upper_Right_Latitude				data.lat_full(3,*)

15		Pixel_Lower_Right_Latitude				data.lat_full(4,*)

These values are the latitude at which the pointing vector from the center and corners of a pixel intersect the target body. If the pointing vector does not intersect the solid surface of the target body (or Saturn’s reference surface) the value is set to –1000.

16		Pixel_Center_Longitude				data.lon_full(0,*)

17		Pixel_Lower_Left_Longitude				data.lon_full(1,*)

18		Pixel_Upper_Left_Longitude				data.lon_full(2.*)

19		Pixel_Upper_Right_Longitude			data.lon_full(3,*)

20		Pixel_Lower_Right_Longitude			data.lon_full(4,*)

Similarly to the latitude planes listed above, these hold the longitude of the intersection of the pointing vector  and the target surface. Again, non-intersecting pixels are defined as –1000.

21		Pixel_Center_Incidence_Angle			data.incidence

The angle between the vector formed by the sun and the point of intersection of the spacecraft pointing vector, and the surface normal vector. If the spacecraft pointing vector does not intersect the target body, the ‘point of intersection’ is taken to be the tangent point.

22		Pixel_Center_Emission_Angle			data.emission

The angle between the surface normal vector and the spacecraft pointing vector.

23		Pixel_Center_Phase_Angle				data.phase

The phase angle based on the emission and incidence angles calculated above.

24 		Pixel_Center_Rayheight				data.rayheight

This is the distance between the tangent point of the pointing vector and the surface of the target. Negative values indicate that the pointing vector intersects the target body. Also called the impact parameter.

25		Pixel_Center_Occultation_Latitude			data.latocc

If the line of sight vector does not intersect the target body, this is the latitude of where the normal vector from the tangent point would intersect the body’s surface. If the vector intersects the body, it is simply the latitude at which the intersection occurs.

26 		Pixel_Center_Occultation_Longitude			data.lonocc

As above, but for longitude.

27 		Pixel_Center_Line_of_Sight_Distance		data.losdist

This is the distance between the spacecraft and the target body, measured along the pointing vector. If the vector does not intersect the planet, it is the distance to the tangent point.

28		Sub_Spacecraft_Longitude				data.subsclon

This is the longitude at which a vector between the spacecraft and the center of mass of the target body intersects the target body surface.

29 		Sub_Spacecraft_Latitude				data.subsclat

As above but for the latitude.

30 		Sub_Solar_Longitude					data.subsolarlon

Similar to the above planes but for the vector between the target body center of mass and the Sun.

31		Sub_Solar_Latitude					data.subsolarlat

As above but for latitude.

32		Spacecraft_Altitude					data.alt

The distance between the spacecraft and the center-of-mass of the target body.

33 		Target_Right_Ascension				data.ratarg

The right ascension coordinate of the target body’s center-of-mass relative to the spacecraft.

34 		Target_Declination					data.dectarg

The declination coordinate of the target body center-of-mass relative to the spacecraft.

35		Target_Phase_Angle					data.phasetarg

36		Target_Incidence_Angle				data.incidencetarg

37		Target_Emission_Angle				data.emissiontar

These angle values are calculated based on the spacecraft and target body with no regard to the pointing of the instrument.

38		Spacecraft_Location, X				data.scloc(0)

The ‘X’ coordinate of the spacecraft relative to the center of the target body, in km.

39		Spacecraft_Location, Y				data.scloc(1)

The ‘Y’ coordinate of the spacecraft relative to the center of the target body, in km.

40		Spacecraft_Location, Z				data.scloc(2)

The ‘Z’ coordinate of the spacecraft relative to the center of the target body, in km.

41		Spacecraft_Velocity, X				data.scvel(0)

The spacecraft velocity in the ‘X’ in km/s.

42		Spacecraft_Velocity, Y				data.scvel(1)

The spacecraft velocity in the ‘Y’ in km/s.

43		Spacecraft_Velocity, Z				data.scvel(2)

The spacecraft velocity in the ‘Z’ in km/s.

44		ET							data.et

Elapsed mission time in seconds.

45		Center_Ring_Plane_Radii				data.rrad_full(0)

46		Lower_Left_Ring_Plane_Radii			data.rrad_full(1)

47		Upper_Left_Plane_Radii				data.rrad_full(2)

48		Upper_Right_Plane_Radii				data.rrad_full(3)

49		Lower_Right_Plane_Radii				data.rrad_full(4)

I’m not completely sure what these values are. Ask Josh Colwell if you are doing any research related to the Rings.

The Following backplane is only written when the image cube is output as an IDL save file, in which case it is a string vector equal in length to the number of readouts.

50		UTC							time

A string giving the date and time of a readout.

51 		Kernels						kernels

A list of all SPICE kernels loaded in IDL memory during cube creation.